# Kalman Filter Dead Reckoning Analysis

Analysis of DR_CORRECTION GPS positions vs DR_DEBUG Kalman-predicted positions.
Three test runs compared: position error, P_diag consistency, and bias growth.

# 1. Per-Test Summary Statistics

## Test: Test 1

Number of DR_CORRECTION events: 145

### Position Error (meters)

| Metric | ΔN (North) | ΔE (East) | Distance |
| --- | --- | --- | --- |
| Mean | 30.32 | -46.39 | 60.01 |
| Median | 35.62 | -50.20 | 66.62 |
| Min | -30.86 | -76.46 | 15.91 |
| Max | 78.65 | -11.94 | 97.58 |
| Std Dev | 28.28 | 20.04 | 25.85 |

### Kalman Consistency (P_diag vs Real Error)

| Metric | Value |
| --- | --- |
| Within 1σ (both N & E) | 0.0% (0/145) |
| Within 2σ (both N & E) | 0.0% (0/145) |

## Test: Test 2

Number of DR_CORRECTION events: 180

### Position Error (meters)

| Metric | ΔN (North) | ΔE (East) | Distance |
| --- | --- | --- | --- |
| Mean | -58.39 | -10.05 | 60.98 |
| Median | -54.18 | -9.84 | 55.58 |
| Min | -109.50 | -45.49 | 2.46 |
| Max | 13.97 | 26.16 | 112.15 |
| Std Dev | 32.90 | 10.78 | 31.45 |

### Kalman Consistency (P_diag vs Real Error)

| Metric | Value |
| --- | --- |
| Within 1σ (both N & E) | 0.0% (0/180) |
| Within 2σ (both N & E) | 1.1% (2/180) |

## Test: Test 3

Number of DR_CORRECTION events: 181

### Position Error (meters)

| Metric | ΔN (North) | ΔE (East) | Distance |
| --- | --- | --- | --- |
| Mean | -36.89 | 36.13 | 54.24 |
| Median | -36.31 | 43.97 | 57.36 |
| Min | -104.15 | -49.91 | 0.29 |
| Max | 20.39 | 75.34 | 128.55 |
| Std Dev | 30.25 | 27.71 | 37.50 |

### Kalman Consistency (P_diag vs Real Error)

| Metric | Value |
| --- | --- |
| Within 1σ (both N & E) | 1.7% (3/181) |
| Within 2σ (both N & E) | 4.4% (7/181) |

# 2. Cross-Test Comparison

## Mean Position Error by Test

| Test | Mean ΔN (m) | Mean ΔE (m) | Mean Distance (m) |
| --- | --- | --- | --- |
| Test 1 | 30.32 | -46.39 | 60.01 |
| Test 2 | -58.39 | -10.05 | 60.98 |
| Test 3 | -36.89 | 36.13 | 54.24 |

## Kalman Consistency by Test

| Test | Within 1σ (%) | Within 2σ (%) |
| --- | --- | --- |
| Test 1 | 0.0% | 0.0% |
| Test 2 | 0.0% | 1.1% |
| Test 3 | 1.7% | 4.4% |

# 3. Bias Growth Analysis

The Kalman filter estimates accelerometer bias (biasN, biasE in m/s²). Bias growth indicates how the filter adapts to sensor drift over time.

## Test: Test 1

| Stage | Corr # | Bias N (m/s²) | Bias E (m/s²) |
| --- | --- | --- | --- |
| Initial | 0 | 0.000000 | 0.000000 |
| Middle | 72 | 0.357500 | -0.341300 |
| Final | 144 | 0.403300 | -0.318400 |

Bias magnitude: 0.000000 → 0.513838 m/s² (Δ = +0.513838)

## Test: Test 2

| Stage | Corr # | Bias N (m/s²) | Bias E (m/s²) |
| --- | --- | --- | --- |
| Initial | 0 | 0.000000 | 0.000000 |
| Middle | 90 | -0.679500 | -0.071900 |
| Final | 179 | -0.679500 | -0.071900 |

Bias magnitude: 0.000000 → 0.683293 m/s² (Δ = +0.683293)

## Test: Test 3

| Stage | Corr # | Bias N (m/s²) | Bias E (m/s²) |
| --- | --- | --- | --- |
| Initial | 0 | 0.000000 | 0.000000 |
| Middle | 90 | -0.244500 | 0.360200 |
| Final | 180 | -0.398100 | 0.420400 |

Bias magnitude: 0.000000 → 0.578982 m/s² (Δ = +0.578982)

# 4. Detailed Correction Data

## Test: Test 1

| Corr# | DR posN | DR posE | GPS posN | GPS posE | ΔN | ΔE | Dist | P_σ_comb |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1.0 | -1.5 | 14.3 | -19.5 | 13.3 | -18.0 | 22.4 | 5.37 |
| 1 | 9.4 | -13.8 | 22.5 | -30.9 | 13.1 | -17.0 | 21.5 | 3.80 |
| 2 | 19.8 | -30.0 | 31.3 | -42.9 | 11.5 | -12.9 | 17.3 | 3.15 |
| 3 | 30.0 | -43.8 | 40.5 | -55.8 | 10.5 | -11.9 | 15.9 | 2.79 |
| 4 | 40.1 | -57.3 | 50.5 | -69.5 | 10.4 | -12.1 | 16.0 | 2.55 |
| 5 | 50.5 | -71.3 | 61.0 | -83.7 | 10.5 | -12.4 | 16.2 | 2.38 |
| 6 | 61.6 | -85.7 | 72.3 | -98.1 | 10.7 | -12.4 | 16.4 | 2.27 |
| 7 | 73.0 | -100.6 | 83.8 | -113.2 | 10.8 | -12.6 | 16.6 | 2.18 |
| 8 | 84.7 | -115.6 | 95.6 | -128.8 | 10.9 | -13.1 | 17.1 | 2.11 |
| 9 | 96.8 | -131.3 | 107.8 | -144.9 | 10.9 | -13.6 | 17.4 | 2.05 |
| 10 | 108.4 | -146.9 | 120.5 | -161.4 | 12.1 | -14.5 | 18.9 | 2.01 |
| 11 | 119.0 | -160.8 | 133.6 | -178.3 | 14.6 | -17.5 | 22.8 | 1.95 |
| 12 | 131.8 | -177.5 | 146.9 | -195.4 | 15.2 | -17.8 | 23.4 | 1.92 |
| 13 | 144.7 | -194.4 | 160.7 | -213.0 | 16.0 | -18.6 | 24.5 | 1.90 |
| 14 | 158.2 | -212.0 | 174.3 | -231.0 | 16.1 | -19.0 | 24.9 | 1.88 |
| 15 | 171.8 | -230.0 | 188.5 | -249.3 | 16.8 | -19.3 | 25.5 | 1.87 |
| 16 | 185.6 | -247.5 | 202.7 | -268.1 | 17.1 | -20.6 | 26.8 | 1.85 |
| 17 | 199.2 | -265.7 | 216.9 | -287.1 | 17.8 | -21.5 | 27.9 | 1.84 |
| 18 | 212.3 | -283.5 | 231.6 | -306.2 | 19.3 | -22.7 | 29.8 | 1.82 |
| 19 | 225.8 | -301.8 | 246.5 | -325.4 | 20.7 | -23.6 | 31.4 | 1.81 |

... and 125 more rows (see JSON for full data).

Last 5 corrections:

| Corr# | DR posN | DR posE | GPS posN | GPS posE | ΔN | ΔE | Dist | P_σ_comb |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 140 | 810.7 | -2823.2 | 881.9 | -2885.5 | 71.3 | -62.3 | 94.6 | 1.60 |
| 141 | 809.6 | -2848.6 | 882.6 | -2909.6 | 73.0 | -61.0 | 95.2 | 1.60 |
| 142 | 809.0 | -2873.6 | 883.5 | -2933.8 | 74.5 | -60.2 | 95.8 | 1.60 |
| 143 | 808.8 | -2898.5 | 884.7 | -2957.6 | 75.9 | -59.2 | 96.2 | 1.60 |
| 144 | 808.0 | -2924.0 | 886.6 | -2981.7 | 78.6 | -57.8 | 97.6 | 1.61 |

## Test: Test 2

| Corr# | DR posN | DR posE | GPS posN | GPS posE | ΔN | ΔE | Dist | P_σ_comb |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0.0 | 0.0 | 14.0 | -45.5 | 14.0 | -45.5 | 47.6 | 5.36 |
| 1 | 10.6 | -44.8 | 20.4 | -68.9 | 9.8 | -24.2 | 26.1 | 3.84 |
| 2 | 18.3 | -75.5 | 26.4 | -91.7 | 8.2 | -16.2 | 18.2 | 3.18 |
| 3 | 24.3 | -102.9 | 32.4 | -115.0 | 8.1 | -12.1 | 14.5 | 2.81 |
| 4 | 30.5 | -128.9 | 38.1 | -138.0 | 7.6 | -9.1 | 11.9 | 2.57 |
| 5 | 36.8 | -154.0 | 43.7 | -161.4 | 7.0 | -7.3 | 10.1 | 2.40 |
| 6 | 42.5 | -178.6 | 48.6 | -184.6 | 6.1 | -6.0 | 8.6 | 2.28 |
| 7 | 48.0 | -202.3 | 52.7 | -207.9 | 4.7 | -5.7 | 7.3 | 2.18 |
| 8 | 53.1 | -226.9 | 56.8 | -231.3 | 3.7 | -4.3 | 5.7 | 2.11 |
| 9 | 57.9 | -250.9 | 60.3 | -254.5 | 2.5 | -3.6 | 4.4 | 2.05 |
| 10 | 62.2 | -275.2 | 64.0 | -278.1 | 1.8 | -2.9 | 3.4 | 2.01 |
| 11 | 66.6 | -299.5 | 67.4 | -302.0 | 0.8 | -2.4 | 2.6 | 1.98 |
| 12 | 71.0 | -323.9 | 69.6 | -325.9 | -1.4 | -2.0 | 2.5 | 1.95 |
| 13 | 73.9 | -343.7 | 71.3 | -349.9 | -2.6 | -6.1 | 6.7 | 1.90 |
| 14 | 76.8 | -367.7 | 72.9 | -373.3 | -4.0 | -5.6 | 6.8 | 1.88 |
| 15 | 79.2 | -392.2 | 74.6 | -397.3 | -4.6 | -5.1 | 6.9 | 1.86 |
| 16 | 81.7 | -416.7 | 75.7 | -421.3 | -6.0 | -4.6 | 7.6 | 1.84 |
| 17 | 84.1 | -441.2 | 76.4 | -445.4 | -7.7 | -4.2 | 8.8 | 1.83 |
| 18 | 86.0 | -465.7 | 76.6 | -469.4 | -9.4 | -3.8 | 10.1 | 1.82 |
| 19 | 87.5 | -489.8 | 76.2 | -493.2 | -11.4 | -3.5 | 11.9 | 1.81 |

... and 160 more rows (see JSON for full data).

Last 5 corrections:

| Corr# | DR posN | DR posE | GPS posN | GPS posE | ΔN | ΔE | Dist | P_σ_comb |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 175 | 302.8 | -4055.4 | 204.0 | -4074.5 | -98.8 | -19.1 | 100.6 | 1.61 |
| 176 | 304.5 | -4074.7 | 208.1 | -4095.9 | -96.5 | -21.3 | 98.8 | 1.59 |
| 177 | 309.1 | -4097.8 | 212.2 | -4117.4 | -96.9 | -19.6 | 98.8 | 1.59 |
| 178 | 313.7 | -4120.6 | 216.1 | -4138.7 | -97.6 | -18.1 | 99.3 | 1.59 |
| 179 | 318.3 | -4143.5 | 220.5 | -4160.6 | -97.8 | -17.1 | 99.3 | 1.59 |

## Test: Test 3

| Corr# | DR posN | DR posE | GPS posN | GPS posE | ΔN | ΔE | Dist | P_σ_comb |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0.0 | 0.0 | 20.4 | -49.9 | 20.4 | -49.9 | 53.9 | 5.36 |
| 1 | 17.6 | -45.0 | 31.4 | -74.5 | 13.8 | -29.6 | 32.6 | 3.82 |
| 2 | 32.0 | -79.3 | 43.1 | -99.0 | 11.1 | -19.6 | 22.5 | 3.17 |
| 3 | 46.0 | -109.1 | 54.7 | -123.0 | 8.7 | -13.9 | 16.4 | 2.81 |
| 4 | 59.5 | -136.2 | 66.1 | -147.0 | 6.6 | -10.8 | 12.7 | 2.57 |
| 5 | 72.8 | -162.9 | 77.9 | -170.9 | 5.1 | -8.0 | 9.5 | 2.40 |
| 6 | 85.6 | -188.4 | 89.0 | -194.7 | 3.4 | -6.3 | 7.1 | 2.28 |
| 7 | 97.6 | -213.8 | 100.4 | -218.5 | 2.8 | -4.7 | 5.5 | 2.19 |
| 8 | 110.0 | -238.5 | 111.1 | -242.1 | 1.1 | -3.5 | 3.7 | 2.12 |
| 9 | 122.0 | -263.5 | 121.9 | -265.5 | -0.1 | -2.0 | 2.0 | 2.06 |
| 10 | 130.3 | -284.3 | 132.9 | -288.8 | 2.6 | -4.5 | 5.2 | 1.99 |
| 11 | 141.3 | -308.7 | 143.7 | -312.1 | 2.4 | -3.4 | 4.2 | 1.96 |
| 12 | 152.4 | -332.8 | 154.6 | -335.6 | 2.1 | -2.8 | 3.5 | 1.93 |
| 13 | 164.0 | -357.1 | 165.3 | -358.9 | 1.3 | -1.8 | 2.2 | 1.90 |
| 14 | 175.3 | -380.7 | 175.9 | -382.0 | 0.6 | -1.3 | 1.5 | 1.88 |
| 15 | 186.8 | -404.7 | 186.7 | -404.9 | -0.1 | -0.3 | 0.3 | 1.87 |
| 16 | 198.2 | -428.5 | 197.3 | -427.8 | -0.9 | 0.7 | 1.2 | 1.85 |
| 17 | 209.4 | -452.0 | 207.8 | -450.2 | -1.6 | 1.8 | 2.4 | 1.83 |
| 18 | 220.6 | -475.8 | 217.9 | -472.9 | -2.7 | 3.0 | 4.0 | 1.82 |
| 19 | 231.5 | -498.9 | 228.1 | -495.6 | -3.4 | 3.3 | 4.7 | 1.81 |

... and 161 more rows (see JSON for full data).

Last 5 corrections:

| Corr# | DR posN | DR posE | GPS posN | GPS posE | ΔN | ΔE | Dist | P_σ_comb |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 176 | 1753.0 | -4004.5 | 1649.3 | -3932.0 | -103.6 | 72.6 | 126.5 | 1.61 |
| 177 | 1752.6 | -4025.0 | 1648.5 | -3949.7 | -104.2 | 75.3 | 128.5 | 1.61 |
| 178 | 1749.6 | -4039.4 | 1648.4 | -3967.7 | -101.3 | 71.7 | 124.1 | 1.59 |
| 179 | 1748.8 | -4059.7 | 1647.1 | -3986.8 | -101.7 | 72.9 | 125.1 | 1.59 |
| 180 | 1747.3 | -4078.9 | 1646.1 | -4006.2 | -101.2 | 72.7 | 124.6 | 1.59 |

# 5. P_diag vs Real Error (Detailed)

Comparison of Kalman filter's estimated position uncertainty (P_diag) against the actual GPS correction error. Ideally, ~68% of errors should fall within 1σ and ~95% within 2σ for a well-tuned filter.

## Test: Test 1

| Corr# | P_σ_N | ¦ΔN¦ | In 1σ? | P_σ_E | ¦ΔE¦ | In 1σ? | Dist |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 3.80 | 13.32 | ✗ | 3.80 | 17.98 | ✗ | 22.38 |
| 1 | 2.69 | 13.06 | ✗ | 2.69 | 17.05 | ✗ | 21.47 |
| 2 | 2.23 | 11.46 | ✗ | 2.23 | 12.92 | ✗ | 17.27 |
| 3 | 1.97 | 10.51 | ✗ | 1.97 | 11.94 | ✗ | 15.91 |
| 4 | 1.80 | 10.39 | ✗ | 1.80 | 12.12 | ✗ | 15.97 |
| 5 | 1.69 | 10.46 | ✗ | 1.69 | 12.38 | ✗ | 16.21 |
| 6 | 1.60 | 10.68 | ✗ | 1.60 | 12.45 | ✗ | 16.40 |
| 7 | 1.54 | 10.78 | ✗ | 1.54 | 12.62 | ✗ | 16.60 |
| 8 | 1.49 | 10.89 | ✗ | 1.49 | 13.15 | ✗ | 17.07 |
| 9 | 1.45 | 10.94 | ✗ | 1.45 | 13.59 | ✗ | 17.45 |
| 10 | 1.42 | 12.11 | ✗ | 1.42 | 14.48 | ✗ | 18.88 |
| 11 | 1.38 | 14.57 | ✗ | 1.38 | 17.51 | ✗ | 22.77 |
| 12 | 1.36 | 15.19 | ✗ | 1.36 | 17.85 | ✗ | 23.43 |
| 13 | 1.35 | 15.97 | ✗ | 1.35 | 18.62 | ✗ | 24.53 |
| 14 | 1.33 | 16.13 | ✗ | 1.33 | 18.97 | ✗ | 24.90 |

... and 130 more rows (see JSON for full data).

## Test: Test 2

| Corr# | P_σ_N | ¦ΔN¦ | In 1σ? | P_σ_E | ¦ΔE¦ | In 1σ? | Dist |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 3.79 | 13.97 | ✗ | 3.79 | 45.49 | ✗ | 47.59 |
| 1 | 2.71 | 9.79 | ✗ | 2.71 | 24.15 | ✗ | 26.06 |
| 2 | 2.25 | 8.16 | ✗ | 2.25 | 16.24 | ✗ | 18.18 |
| 3 | 1.99 | 8.07 | ✗ | 1.99 | 12.10 | ✗ | 14.55 |
| 4 | 1.82 | 7.62 | ✗ | 1.82 | 9.12 | ✗ | 11.88 |
| 5 | 1.70 | 6.98 | ✗ | 1.70 | 7.35 | ✗ | 10.13 |
| 6 | 1.61 | 6.06 | ✗ | 1.61 | 6.05 | ✗ | 8.56 |
| 7 | 1.54 | 4.67 | ✗ | 1.54 | 5.65 | ✗ | 7.33 |
| 8 | 1.49 | 3.71 | ✗ | 1.49 | 4.32 | ✗ | 5.69 |
| 9 | 1.45 | 2.48 | ✗ | 1.45 | 3.64 | ✗ | 4.40 |
| 10 | 1.42 | 1.77 | ✗ | 1.42 | 2.90 | ✗ | 3.40 |
| 11 | 1.40 | 0.78 | ✓ | 1.40 | 2.44 | ✗ | 2.56 |
| 12 | 1.38 | 1.43 | ✗ | 1.38 | 2.00 | ✗ | 2.46 |
| 13 | 1.35 | 2.64 | ✗ | 1.35 | 6.11 | ✗ | 6.66 |
| 14 | 1.33 | 3.96 | ✗ | 1.33 | 5.55 | ✗ | 6.82 |

... and 165 more rows (see JSON for full data).

## Test: Test 3

| Corr# | P_σ_N | ¦ΔN¦ | In 1σ? | P_σ_E | ¦ΔE¦ | In 1σ? | Dist |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 3.79 | 20.39 | ✗ | 3.79 | 49.91 | ✗ | 53.91 |
| 1 | 2.70 | 13.75 | ✗ | 2.70 | 29.59 | ✗ | 32.63 |
| 2 | 2.24 | 11.05 | ✗ | 2.24 | 19.63 | ✗ | 22.53 |
| 3 | 1.98 | 8.70 | ✗ | 1.98 | 13.89 | ✗ | 16.39 |
| 4 | 1.81 | 6.63 | ✗ | 1.81 | 10.78 | ✗ | 12.66 |
| 5 | 1.70 | 5.08 | ✗ | 1.70 | 8.01 | ✗ | 9.48 |
| 6 | 1.61 | 3.38 | ✗ | 1.61 | 6.26 | ✗ | 7.11 |
| 7 | 1.55 | 2.81 | ✗ | 1.55 | 4.74 | ✗ | 5.51 |
| 8 | 1.50 | 1.14 | ✓ | 1.50 | 3.52 | ✗ | 3.70 |
| 9 | 1.46 | 0.11 | ✓ | 1.46 | 1.96 | ✗ | 1.96 |
| 10 | 1.41 | 2.64 | ✗ | 1.41 | 4.49 | ✗ | 5.21 |
| 11 | 1.39 | 2.36 | ✗ | 1.39 | 3.44 | ✗ | 4.17 |
| 12 | 1.36 | 2.12 | ✗ | 1.36 | 2.83 | ✗ | 3.54 |
| 13 | 1.35 | 1.34 | ✓ | 1.35 | 1.79 | ✗ | 2.24 |
| 14 | 1.33 | 0.63 | ✓ | 1.33 | 1.32 | ✓ | 1.47 |

... and 166 more rows (see JSON for full data).

# 6. Summary & Conclusions

Total corrections analyzed across all tests: 506

Overall mean position error: 58.29 m

Overall median position error: 61.16 m

Overall max position error: 128.55 m

Overall min position error: 0.29 m

Overall within 1σ: 0.6% (3/506)

Overall within 2σ: 2.0% (10/506)

Note: For a well-tuned Kalman filter, ~68% of errors should fall within 1σ and ~95% within 2σ. Values significantly below these thresholds suggest the filter is overconfident (P_diag too small); values above suggest the filter is too conservative (P_diag too large).